- For configuration of the transmitter, you’ll need a mobile phone and an additional Meshtastic node connected to the “api” channel.

- To switch into the configuration mode, push the button on below of the transmitter (works only in sleep mode):
MeshBee stops making measurements of the sensors and creates a Wifi Access Point (AP):
SSID: MeshBee		Password: MeshBee123
Connect to WiFi Meshbee and deactivate the mobile data on the phone!
In case of 15 minutes without a client connected, the ESP will reset and turn back to normal operation mode.

- For updating the software, navigate to:
192.168.4.1/update
Here you can upload the precompiled firmware <xyz>.bin.

- For changing the sensors and for entering any command, navigate to:
192.168.4.1/webserial

  - command: list
lists all sensors

  - command: sleep
shows the time period for sleep mode

  - command: example (example sensor)
shows an example of sensor configuration. Watch below.

  - command: exsleep (example sleep time)
shows an example how to setup the time period for sleep mode. Watch below.

  - command: reset
resets MeshBee, shuts off the AP and starts making measurements

  - command: erase
erases all configuration data from the permanent memory

  - command: store
Saver all configuration data in the permanent memory. DONT’T FORGET AFTER INSERTING OR MODIFICATION OF ANY DATA!

  - command: reload
reloads all configuration data from the permanent memory in case of messing up modifications. Works only before the “store” command.

  - Command (change calibration data):
 {“sensor”:<a>,”sensDHT22”: <b>,”sensHX711”: <c>,”offset”:<d>,”factor”:<e>}
<a>: (0 – 7) sensor number
<b>: (true / false) DHT22 present / not present
<c>: (true / false) load cell present / not present
<d>: offset value i.e. -297000
<e>: factor value i.e. 213462
complete example: 
{“sensor”:1,”sensDHT22”: true,”sensHX711”: true,”offset”:-297000,”factor”:213462}
Watch above how to get an example.
To set up easily a sensor, simply scan the QR code of the sensor and set up the values here accordingly.

  - Command (modificate and save permanently the time period for sleep mode):
 {“sleep_time”:<a>}
<a>: sleep time in minutes
complete example: 
{“sleep_time”:60}
Watch above on how to get an example.

  - Commando: ver
Software version

- For each sensor with DHT22 and/or HX711 set up in the configuration, MeshBee transmits a Meshtastic messagge. Also, if there is an error with the sensor reading or the sensor is not connected.

- For returning to taking measurements of the sensor (normal mode) type the command “reset” or push the button.

- For checking the set-up sensors, connect to the Meshtastic app with a portable node (with the “ape” channel) and check the messages in normal mode.
If you connect directly to the transmitter node, you will not see the messages.

- If in normal mode the transmitter sends the value “null”, then there is an issue with the sensor and it was not possible to read it. Also, when you get a message as: 
“ sensor <X> error on <YYYYY>”