- For configuration of the receiver, you’ll need a mobile phone and an additional Meshtastic node connected to the “api” channel.

- For configuration connect to Wifi Access Point (AP):
SSID: MeshBeeR		Password: MeshBee123
Connect to WiFi Meshbee and deactivate the mobile data on the phone!

- For updating the software, navigate to:
192.168.4.1/update
Here you can upload the precompiled firmware <xyz>.bin.

- For changing the sensors and for entering any command, navigate to:
192.168.4.1/webserial

  - command: list
lists all sensors

  - command: example (example node ID)
shows an example of the configuration of a node ID. Watch below.

  - command: reset
resets the receiver

  - command: test
sends test message to the backend

  - command: erase
erases all configuration data from the permanent memory
Also the WiFi access data

  - Command (change node ID):
 {“nodeID”:”<aaaa>”}
<aaaa>: nodeID
complete example: 
{“nodeID”:”388D”}
Watch above how to get an example.

  - Commando: ver
Software version