Manual for the calibration of the sensors

- Connect your mobile phone to the Meshtastic node of the transmitter and change the Lora setup to “SHORT_FAST”. So, you don’t spoil the database with uncalibrated data.

- Connect your mobile phone to a portable Meshtastic node with the APE channel and change the Lora setupt to “SHORT_FAST”

- Take off all weights from the sensor

- Push the button on below of the transmitter

- Connect to the MeshBeeT Wifi. Password: MeshBee123

- With your browser goto 192.168.4.1/webserial

- Insert the calibration data with offset 0 and factor 1
{“sensor”:<x>,”sensDHT22”:true,”sensHX711”:true,”offset”:0,”factor”:1}
alternatively take the output of command “example”

- Type command “store”

- Type command “reset”

- Now the sensor resets and sends values. You will get them as message in your Meshtastic APP.

- Memorize the given weight value.

- Wait about 15 seconds until the transmitter changes to deep sleep mode.

- Push the button again and connect to webserial

- Insert the calibration data and take as “offset” value the given weight.

- Execute again the “store” and “reset command”

- If the new weight is around 0, it was successful

- Now put on a known weight and push the button and again ca. after 10 seconds.

- Take the new given weight and divide it by the known weight.

- Push the button again and go to webserial

- Now insert the result of the division as the new “factor”.

- After “store” and “reset” it should give the real weight and the sensor is calibrated.

- Change the Lora setup on the transmitter and you mobile node to “MEDIUM_FAST”